Training Syllabus to Instruct/Prepare for 
the ASE Transit Bus Electrical/Electronics Test

Abstract: This Recommended Practice provides guidelines for establishing a standardized bus maintenance training related to the ASE certification program syllabus for the electrical/electronics systems used in transit buses and coaches.

Keywords: Automotive Service Excellence (ASE) H6, bus, certification, electronics, training, transit

Summary: This Recommended Practice provides transit bus maintenance training and transit bus maintenance departments with typical information to instruct/prepare transit bus technicians and mechanics for the ASE H6 Transit Bus Electrical/Electronic Test and to evaluate, develop or enhance current training programs for the diagnosis, repair and maintenance of transit bus electrical/electronic systems. Individual operating agencies should modify these guidelines to accommodate their specific equipment and mode of operation.

Scope and purpose: This Recommended Practice reflects the consensus of the APTA Bus Standards Program members in conjunction with transit labor organizations, including ATU and TWU, on the subject material, manuals and textbooks, test equipment, methods and procedures that have provided the best performance record based on the experiences of those present and participating in meetings of the program task forces and working groups. This document covers basic, intermediate and advanced copper wired electrical circuits, programmable logic controllers (PLC) and multiplexing electrical/electronic systems used in transit buses as related to the H6 ASE Certification Test. Although electrical circuits may be referenced for peripheral system electronics used in the control of engines, transmissions, wheelchair lifts, fire suppression systems, etc., the testing for these systems shall be separate and covered by a different test series. APTA recommends the use of this document by organizations that have a training department or conduct training for the maintenance of transit buses, organizations that contract with others for transit bus maintenance training, and organizations that influence how training for transit bus maintenance is conducted.

Contents

Participants

The American Public Transportation Association greatly appreciates the contributions of the Bus Maintenance Training Working Group, which provided the primary effort in the drafting of this Recommended Practice.

At the time this standard was completed, the working group included the following members:

Co-Chair-Bob Hykaway ATU

Co-Chair-Dennis Cristofaro CTA

1.	Learning environment	1

2.	Computer skills	1

3.	Course descriptions and objectives....................................1

4.	Exam requirements	2

5.	ASE test content summary…………………………………....3

References	4

Abbreviations and Acronyms	4

Appendix A- Transit Bus Electrical/Electronics Learning

Objectives………………………………………………..……….…….5

Appendix B- ASE Electrical/Electronics Task List…………..…..14

Appendix C- Sample Curriculum………………….…….…………17

# Learning environment

For best application of this Recommended Practice, a combination of classroom lectures, mentoring, practical training, and practice tests should be included in the training program.

# Computer skills

Basic computer skills are now a standard for transit bus technicians. Basic skills and knowledge in the operation of a computer in a Microsoft Windows environment is essential.

# Course learning objectives

The Learning Objectives listed below have been developed through a Labor-Management Committee of Subject Matter Experts (SME).  The Learning Objective Levels represent 100 (Introductory), 200 (Intermediate), and 300 (Advanced).  Within each level the Learning Objectives are organized in the recommended order of instruction.  When a transit bus mechanic demonstrates proficiency in the learning objectives they should be capable of attaining ASE Transit Bus Technician Certification.

Module A-1, A-2: General Electrical Diagnosis 101, 201

The objective of these courses is to familiarize the employee with the basics of electrical diagnosis including:  pertinent electrical theories and laws, schematic reading, safety issues, applicable tools/equipment, and circuit protection.  More advanced aspects are covered in the 201 level course where emphasis is placed on advanced electrical theories, testing equipment, advanced schematic reading, ladder logic diagrams, multiplexing systems, and circuits, solenoids, diodes, resistors, and capacitors.

## Module B-1: Battery 101

The objective of this course is to familiarize the employee with bus battery basics including safety, testing, cleaning and inspection, replacement, charging, and jump starting a bus.  Employees will be taught to restore battery and related components to OEM specifications.

## Module C-1, C-2:  Starting Systems 101, 201

The objective of these courses is to familiarize the employee with the starting system diagnosis and repair and improving employee skills with regards to advance repairs.  Emphasis will be placed on inspection, component identification, circuit component replacement, starter circuit wiring, distinguishing between mechanical and electrical causes of starter problems, and restoring starting systems to OEM specifications

Module D-1, D-2:  Charging Systems 101, 201

The objective of these courses is to familiarize the employee with the basics of charging systems, component identification, diagnosis, inspection, testing, and troubleshooting.  Focus will be placed on adjusting, replacing, and removing alternators and checking, adjusting, and replacing voltage regulators as well as proper testing and inspection procedures for proper diagnosis and repair.

## Module E-1:  Lighting Systems 101

The objective of this course is to instruct the employee how to read wiring diagrams, perform visual inspections, and replace various interior and exterior lights and lighting circuits.  Emphasis is placed on restoring lighting systems to OEM specifications.

## Module F.G-1:  Wiring Connections, and Connectors 101

Participants will receive classroom instruction where a qualified instructor will review the basics of wiring, connections, and connectors as it relates to gauges, warning devices, horns, wipers, and HVAC components.  Participants will receive instruction on related wiring schematics and how to solder and splice connections.

## Module F.G-2, F.G-3:  Gauges, Warning Devices, and Related Systems 201, 301

The objective of these courses is to familiarize the employee with how to use wiring diagrams to troubleshoot, diagnose, inspect, and repair/replace complex electrical circuits, how to properly use test equipment, and how maintain various switches.  Emphasis is placed on developing a high understanding of data communications and multiplex systems.  The 301 level course is focused on fire suppression system maintenance, diagnosis, and repair as well as multiplexing system troubleshooting, inspection, and maintenance.

# Exam requirements

The minimum acceptable grade to pass the course and all practical tests is 75 percent. Students must pass written tests with a minimum grade of 80 percent.

# ASE Test Content Summary

| TABLE 1 Specifications for Transit Bus Electrical/Electronics Test |  |  |
| --- | --- | --- |
| Content Area | Questions in Test | Percent of Test |
| A. General Electrical Diagnosis | 16 | 32% |
| B. Battery Diagnosis and Repair | 5 | 10% |
| C. Starting System Diagnosis and Repair | 7 | 14% |
| D. Charging System Diagnosis and Repair | 7 | 14% |
| E. Lighting Systems Diagnosis and Repair 1. Headlights, Daytime Running Lights, Parking, Clearance, Tail, Interior and Dash Lights (5) 2. Stoplights, Turn Signals, Hazard Lights and Backup Lights (3) | 7 | 14% |
| F. Gauges and Warning Devices Diagnosis and Repair | 4 | 8% |
| G. Related Systems | 4 | 8% |
| Total | 50 | 100% |

Please see Appendix B for the full list of ASE course tasks that should be covered during the training course.

# References

National Institute for Automotive Service Excellence (ASE) website. http://www.ase.com/

# Abbreviations and acronyms

A/C	air conditioning

APTA	American Public Transportation Association

ASE	Automotive Service Excellence

ATU	Amalgamated Transit Union

CPU	central processing unit

DIO	digital input/output

DMM	digital multimeter

DRL	daytime running lights

DSM	digital scope meter

GMM	graphing multimeter

HCNC	high-speed cell net controller

I/O	input/output

LED	light-emitting diode

MBC	main bus controller

mux	multiplexer

OEM	original equipment manufacturer

PLC	programmable logic controller

PM	Preventative Maintenance

PPE	personal protective equipment

PTC	positive temperature coefficient

SAE	SAE International, formerly the Society of Automotive Engineers

TWU	Transit Workers Union

| Appendix A Transit Bus Electrical/Electronics Learning Objectives |  |
| --- | --- |
| 101 SAFETY & PROTECTION |  |
| Learning Objectives | ASE Task Reference |
| Follow proper electrical safety procedures and use proper personal protective equipment (PPE) | A17 |
| 101 SCHEMATICS |  |
| Learning Objectives | ASE Task Reference |
| Read basic wiring diagrams | A12, E22, E23, E24, F1, F2, G2, G3, G5, G6, G7 |
| Identify symbols used in electrical schematics | A12 |
| Read single-circuit electrical schematics | A12 |
| 101 BATTERIES |  |
| Learning Objectives | ASE Task Reference |
| Safely handle batteries and acid | B3 |
| Inspect batteries | B3, C5, D8, D9 |
| Check batteries for water | B1, B2, B3, B4, B5, B6 |
| Clean and check battery and terminal connections | B3, C5 |
| Measure battery voltage/surface charge | B2, D8, D9 |
| Check battery box construction for stress cracks/welds | B4 |
| Replace batteries | B3 |
| 101 CIRCUIT PROTECTION |  |
| Learning Objectives | ASE Task Reference |
| Inspect and test circuit breakers | A9 |
| Replace circuit breakers | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Inspect and test fuses | A9 |
| Replace fuses | E23, F3 |
| 101 STARTING |  |
| Learning Objectives | ASE Task Reference |
| Inspect starters (motors and solenoids) | A11, C2, C3 |
| Perform voltage drop tests on starter circuits using tester unit | C1 |
| Replace starter circuit relays | A11, C2, C3 |
| Replace starter circuit solenoids | A11, C3 |
| Replace starter circuit switches | C2, C3 |
| Replace starter motor | C4 |
| 101 CHARGING |  |
| Learning Objectives | ASE Task Reference |
| Adjust or replace alternator components (belts, gears, fans, etc.) | A17, D12 |
| Remove and replace alternators | D6 |
| 101 LIGHTING |  |
| Learning Objectives | ASE Task Reference |
| Visually check wiring for exterior lights | E11, E14, E16 |
| Replace headlights, LED, fluorescent, and incandescent lights | E12 |
| Visually check wiring for interior lights | E16 |
| 101 ELECTRICAL THEORY |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate knowledge of basic electrical theories and laws | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 102 BATTERIES |  |
| Learning Objectives | ASE Task Reference |
| Measure battery acid specific gravity | B2 |
| Perform battery load and capacity tests on batteries | A8, B1 |
| Charge batteries in vehicles | B5 |
| Jump-start bus using jumper cables | B6 |
| Jump-start bus using auxiliary power supply | B6 |
| 102 CIRCUIT PROTECTION |  |
| Learning Objectives | ASE Task Reference |
| Diagnose condition of fuses, circuit breakers, and switches | A9 |
| 102 SWITCHES, RELAYS & SOLENOIDS |  |
| Learning Objectives | ASE Task Reference |
| Identify symbols used for various switches, relays, and solenoids | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Identify different types and purposes of relays, switches, and solenoids | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Inspect and test solenoids, switches and relays | A11 |
| Inspect micro-switches | A16 |
| Replace various switches, relays and solenoids | A16, E14 |
| 102 CHARGING |  |
| Learning Objectives | ASE Task Reference |
| Check and adjust voltage regulator set points | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 102 WIRING, CONNECTIONS & CONNECTORS |  |
| Learning Objectives | ASE Task Reference |
| Replace/assemble connectors | C5, D7, E13, E14, E15, E16, E21, E23, E24, F3, F4, G1, G3, G4, G5, G6 |
| Use heat shrinking and crimping equipment | G1, G2, G3, G4, G5, G6, G7 |
| 102 LIGHTING |  |
| Learning Objectives | ASE Task Reference |
| Replace interior or exterior lighting circuits | E14 |
| Inspect electrical circuits | F3, F4 |
| 103 SCHEMATICS |  |
| Learning Objectives | ASE Task Reference |
| Determine which schematic is related to a specific bus system | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 103 SWITCHES, RELAYS & SOLENOIDS |  |
| Learning Objectives | ASE Task Reference |
| Inspect and test relays | A11 |
| Check, repair or replace electrical relays | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17, E23, G1 |
| 103 STARTING |  |
| Learning Objectives | ASE Task Reference |
| Inspect various related relay on starting circuits | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 103 CHARGING |  |
| Learning Objectives | ASE Task Reference |
| Replace voltage regulators | D5 |
| 103 LIGHTING |  |
| Learning Objectives | ASE Task Reference |
| Replace ballast for interior lights | E16 |
| 103 TEST EQUIPMENT |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proficient use of digital multimeters and other common test equipment | A2, A4 |
| Perform tests for voltage draws and drops using a digital multimeter | A3 |
| 103 ELECTRO-PNEUMATIC |  |
| Learning Objectives | ASE Task Reference |
| Inspect electrically operated air equipment | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 104 ELECTRICAL CIRCUITS |  |
| Learning Objectives | ASE Task Reference |
| Repair or replace any electrical circuit components | A17, E14, E15, E22, E23, E24, F5, G1, G5, G6, G7 |
| 104 WIRING, CONNECTIONS & CONNECTORS |  |
| Learning Objectives | ASE Task Reference |
| Solder connections | G1, G2, G3, G4, G5, G6, G7 |
| Splice connections | G1, G2, G3, G4, G5, G6, G7 |
| 104 ELECTRO-PNEUMATIC |  |
| Learning Objectives | ASE Task Reference |
| Replace electrically operated air equipment | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 201 SCHEMATICS |  |
| Learning Objectives | ASE Task Reference |
| Draw basic wiring diagrams | A12 |
| Use wiring diagrams to troubleshoot electrical problems | A12 |
| Use wiring diagrams to troubleshoot problems with gauges and warning devices | F1, F2, F3, F4, F5 |
| 201 MULTIPLEXING |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate the ability to read and interpret ladder logic diagrams. | A13, A14 |
| Use LEDs to troubleshoot system | A14 |
| Explain the functions of the multiplex controller and its related components | A14 |
| Use ladder logic diagrams to troubleshoot a multiplex system | A14 |
| Read PLC addressing | F1, F2, F3, F4, F5 |
| Use logic diagrams to troubleshoot a PLC system | F1, F2, F3, F4, F5 |
| Identify symbols used for multiplexing inputs and outputs | F1, F2, F3, F4, F5 |
| Replace multiplexing input and output cards | F1, F2, F3, F4, F5 |
| Replace HCNC and DIO modules | F1, F2, F3, F4, F5 |
| Use multiplex software | F1, F2, F3, F4, F5 |
| Replace control rack | F1, F2, F3, F4, F5 |
| 201 STARTING |  |
| Learning Objectives | ASE Task Reference |
| Distinguish between mechanical and electrical causes of starter problems | C4, C6 |
| Replace starter circuit wiring | C1, C2, C3, C4, C5, C6 |
| Inspect and diagnose starting systems | C1, C2, C3, C4, C5, C6 |
| 201 CHARGING |  |
| Learning Objectives | ASE Task Reference |
| Diagnose cause of various charging conditions and determine proper repair procedure | D1 |
| Inspect overall charging system operation | D3 |
| Inspect and repair connectors and wires in charging circuits | D7, D8 |
| Perform alternator output tests (amp draw test) using tester unit | D1, D3 |
| Diagnose charging systems using fault tree chart | D1, D2, D3, D4, D5, D6, D7, D8, D9 |
| Perform alternator voltage drop tests using tester unit | D4 |
| Inspect and troubleshoot power supply | F1, F2, F3, F4, F5 |
| 201 ELECTRICAL CIRCUITS |  |
| Learning Objectives | ASE Task Reference |
| Diagnose circuit malfunctions (short circuits, grounded circuits and open circuits) | A1, A2, A7, E22, E23, E24, F1, F2, G2, G3, G5, G6, G7 |
| Diagnose, inspect and troubleshoot complex traditional electrical circuits (non electronic) | F1, F2, F3, F4, F5 |
| Replace/rewire electrical power circuits | G1, G2, G3, G4, G5, G6, G7 |
| 201 WIRING, CONNECTIONS & CONNECTORS |  |
| Learning Objectives | ASE Task Reference |
| Inspect wiring on engines | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Remove and replace electrical connectors and terminals | G1, G2, G3, G4, G5, G6, G7 |
| 201 LIGHTING |  |
| Learning Objectives | ASE Task Reference |
| Diagnose lighting systems and circuits | E13, E14, E15, E21, E22, E23, E24, F4 |
| 201 DOORS |  |
| Learning Objectives | ASE Task Reference |
| Remove, replace or adjust sensitive door edge wiring | G1, G2, G3, G4, G5, G6, G7 |
| Inspect door circuits | G1, G2, G3, G4, G5, G6, G7 |
| Inspect door edge wiring | G1, G2, G3, G4, G5, G6, G7 |
| Replace door system wiring | G1, G2, G3, G4, G5, G6, G7 |
| Distinguish between pneumatic, mechanical and electrical causes of door problems | G1, G2, G3, G4, G5, G6, G7 |
| Replace or repair/rewire door circuits | G1, G2, G3, G4, G5, G6, G7 |
| 201 TEST EQUIPMENT |  |
| Learning Objectives | ASE Task Reference |
| Use diagnostic software to troubleshoot equipment | A1 |
| Use ammeter to inspect motor | A4 |
| Use digital multimeters to check circuit voltage amperage and resistance | A6, E22, E23, E24, F1, F2, G2, G3, G5, G6, G7 |
| Operate Windows-based computer for diagnostic purposes | A15 |
| Demonstrate basic PC knowledge | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 201 ELECTRONICS & DATA COMMUNICATIONS |  |
| Learning Objectives | ASE Task Reference |
| Inspect and test resistors | A6, A10 |
| Inspect and test diodes | A10 |
| Inspect and test capacitors | A10 |
| Replace electronic modules | F1, F2, F3, F4, F5 |
| 201 ELECTRONIC THEORY |  |
| Learning Objectives | ASE Task Reference |
| Explain the operation of a suppression diode and test a suppression diode | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Describe the difference between digital and analog signals | F1, F2, F3, F4, F5 |
| Describe purpose of network protocol CAN/J1939 and J1708 | F1, F2, F3, F4, F5 |
| 201 SOLENOIDS |  |
| Learning Objectives | ASE Task Reference |
| Replace solenoids | A11 |
| Describe function of power equalizer | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Replace gateway module (SAE network to mux) | F1, F2, F3, F4, F5 |
| 202 SCHEMATICS |  |
| Learning Objectives | ASE Task Reference |
| Use drawings to assemble bus electrical equipment for new circuits (retrofits) | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 202 MULTIPLEXING |  |
| Learning Objectives | ASE Task Reference |
| Troubleshoot multiplex controller for communication problems | A1 |
| Diagnose, inspect and troubleshoot input and output circuits in multiplex systems | A13, A14 |
| Repair faults through reading ladder logic diagrams | A14 |
| Troubleshoot electrical faults by analyzing combinations of inputs and outputs | A14 |
| Inspect and troubleshoot remote I/O blocks | F1, F2, F3, F4, F5 |
| Replace remote I/O blocks | F1, F2, F3, F4, F5 |
| Cross reference ladder diagrams and schematics to locate ladder rungs related to particular systems | F1, F2, F3, F4, F5 |
| Check multiplex control modules | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Replace multiplex control modules | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Describe the functions of the multiplex controller and its related components | F1, F2, F3, F4, F5 |
| Describe function of EM70 microprocessor based controls | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Replace central processing unit (CPU) card | F1, F2, F3, F4, F5 |
| 202 ELECTRICAL CIRCUITS |  |
| Learning Objectives | ASE Task Reference |
| Repair interlock systems | G1, G2, G3, G4, G5, G6, G7 |
| Replace current limiters/PTC | F1, F2, F3, F4, F5 |
| 202 ELECTRONICS & DATA COMMUNICATIONS |  |
| Learning Objectives | ASE Task Reference |
| Replace diodes | A10 |
| Replace resistors | A10 |
| Replace capacitors | A10 |
| Diagnose and replace faulty circuit boards | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 202 SENSORS, SENDERS & PROBES |  |
| Learning Objectives | ASE Task Reference |
| Identify sensors, senders and probes | A16, F5 |
| Determine expected sensor output | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Determine sensor air gap | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 202 SWITCHES |  |
| Learning Objectives | ASE Task Reference |
| Identify proper settings and calibrate address/dip switches | F1, F2, F3, F4, F5 |
| Repair or replace magnetic, optical and/or half-effect proximity switches | F1, F2, F3, F4, F5 |
| 202 TEST EQUIPMENT |  |
| Learning Objectives | ASE Task Reference |
| Use service tools to diagnose communication link problems | F1, F2, F3, F4, F5 |
| Use fault codes to diagnose and repair systems | F1, F2, F3, F4, F5 |
| 202 WIPERS |  |
| Learning Objectives | ASE Task Reference |
| Maintain wiper motors | G3, G4 |
| 203 SCHEMATICS |  |
| Learning Objectives | ASE Task Reference |
| Reference bus maintenance manuals to locate and use specific electrical schematics | G1, G2, G3, G4, G5, G6, G7 |
| 203 WIRING, CONNECTIONS & CONNECTORS |  |
| Learning Objectives | ASE Task Reference |
| Repair or create wiring harnesses | G1, G2, G3, G4, G5, G6, G7 |
| 203 MULTIPLEXING |  |
| Learning Objectives | ASE Task Reference |
| Inspect/Troubleshoot input and output cards | F1, F2, F3, F4, F5 |
| 203 TEST EQUIPMENT |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate use of lab scope/oscilloscope/graphing multimeter | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 203 ELECTRONIC THEORY |  |
| Learning Objectives | ASE Task Reference |
| Describe magnetic inductance | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Describe basic electronics theory of components | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| Describe waveforms (pulse width, frequency/hertz, duty cycle) | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16, A17 |
| 203 ELECTRONICS & DATA COMMUNICATIONS |  |
| Learning Objectives | ASE Task Reference |
| Inspect and troubleshoot CPU card | F1, F2, F3, F4, F5 |
| Inspect and troubleshoot Gateway module | F1, F2, F3, F4, F5 |
| Inspect, troubleshoot and repair data communications network | F1, F2, F3, F4, F5 |
| Check, repair or replace data communications cable | F1, F2, F3, F4, F5 |
| Inspect and troubleshoot HCNC and DIO modules | F1, F2, F3, F4, F5 |
| Use code scanner and adapter card to troubleshoot system | F1, F2, F3, F4, F5 |
| 301 MULTIPLEXING |  |
| Learning Objectives | ASE Task Reference |
| Troubleshoot and maintain multiplexing systems | F1, F2, F3, F4, F5 |
| Inspect/Troubleshoot main bus controller (MBC) and PMS modules | F1, F2, F3, F4, F5 |
| Check/repair/replace control rack | F1, F2, F3, F4, F5 |
| 301 FIRE SUPPRESSION |  |
| Learning Objectives | ASE Task Reference |
| Maintain fire suppression electrical systems | G1, G2, G3, G4, G5, G6, G7 |
| Diagnose and repair or replace VMU unit | G1, G2, G3, G4, G5, G6, G7 |

## Appendix B

## ASE Course Task List

## General electrical diagnosis

Task 1: Verify operator complaint, reproduce the condition (including intermittent problems), and/or road test vehicle; determine necessary action.

Task 2: Check continuity in electrical/electronic circuits using appropriate test equipment.

Task 3: Check applied voltages, circuit voltages, and voltage drops in electrical/ electronic circuits using a digital multimeter (DMM), or a graphing multimeter (GMM).

Task 4: Check current flow in electrical/electronic circuits and components using an ammeter, DMM, clamp-on ammeter, or GMM.

Task 5: Check electronic circuit waveforms using an oscilloscope; interpret readings and determine needed repairs.

Task 6: Check resistance in electrical/electronic circuits and components using an ohmmeter, DMM or GMM.

Task 7: Locate shorts, grounds and opens in electrical/electronic circuits.

Task 8: Diagnose battery drain problems with the master/key switch off.

Task 9: Inspect and test fusible links, circuit breakers, solid-state current limiters and fuses; replace as required.

Task 10: Inspect and test spike suppression diodes/resistors and capacitors; replace as required.

Task 11: Inspect and test relays and solenoids; replace as required.

Task 12: Read and interpret electrical schematic diagrams and symbols.

Task 13: Read and interpret ladder logic diagrams.

Task 14: Diagnose and repair computer communication multiplex systems; determine needed repairs.

Task 15: Using a laptop computer, establish communication with a multiplex control system.  Verify that the needed ladder logic inputs are active to control an individual/specific ladder logic output.

Task 16: Remove, replace, and adjust electrical/electronic switches, sensors, and other electrical/electronic components.

Task 17: proper care and handling of electrical/electronic components.

## Battery diagnosis and repair

Task 1: Perform battery tests (load, capacitance and specific gravity); determine needed repairs.

Task 2: Determine battery state of charge by measuring terminal post voltage using a DMM.

Task 3: Inspect, clean, and service battery, cables, terminal connections and disconnects; replace as required.

Task 4: Inspect, clean and repair battery boxes, mounts and hold downs; replace as required.

Task 5: Charge battery(s), using slow or fast charge method as appropriate.

Task 6: Jump-start a transit bus using jumper cables and a booster battery or auxiliary power supply.

## Starting system diagnosis and repair

Task 1: Perform starter circuit voltage drop tests; determine needed repairs.

Task 2: Inspect and test components of the starter control circuit (key switch, push button and/or magnetic switch and wires); replace as required.

Task 3: Inspect and test starter relays and solenoids/switches; replace as required.

Task 4: Remove and replace starter; inspect flywheel ring gear or flex plate.

Task 5: Inspect, clean, repair or replace cranking circuit battery cables and connectors.

Task 6: Differentiate among electrical, multiplex or mechanical problems that cause a slow cranking, no cranking, extended cranking or cranking noise conditions.

## Charging system diagnosis and repair

Task 1: Diagnose the cause of a no-charge, low-charge, or overcharge condition; determine needed repairs.

Task 2: Inspect and adjust alternator drive belts/gears, pulleys, fans, mounting brackets, and tensioners; replace as required.

Task 3: Perform charging system output tests (12-volt and 24-volt); determine needed repairs.

Task 4: Perform charging circuit voltage drop tests; determine needed repairs.

Task 5: Test, adjust or replace voltage regulator.

Task 6: Maintain, remove, and replace alternator.

Task 7: Inspect, repair or replace charging circuit connectors and wires.

Task 8: Check battery equalizer output, check wiring and mounting; determine needed repairs.

Task 9: Verify operation of charging system circuit monitor; determine needed repairs.

## Lighting systems diagnosis and repair

### Headlights, daytime running lights, parking, clearance, tail, interior and dash lights

Task 1: Diagnose the cause of brighter than normal, intermittent, dim, or no headlight and daytime running light (DRL) operation.

Task 2: Test, aim, and replace headlights.

Task 3: Test headlight and dimmer switches, wires, connectors, terminals, sockets, relays and control components; repair or replace as required.

Task 4: Inspect, test and repair parking, clearance and taillight circuit switches, bulbs, sockets, connectors, terminals, relays, wires and light-emitting diodes (LEDs); replace as required.

Task 5: Inspect, test and repair dash light circuit switches, bulbs, sockets, connectors, terminals, wires and printed circuits; replace as required.

Task 6: Inspect, test and repair interior and exterior light circuit switches, bulbs, sockets, connectors, terminals, ballasts and wires; replace as required.

### Stoplights, turn signals, hazard lights and backup lights

Task 1: Inspect and test stoplight circuit switches, bulbs, sockets, connectors, terminals, relays, control components and wires; repair or replace as required.

Task 2: Diagnose the cause of turn signal and hazard flasher light system malfunctions; determine needed repairs.

Task 3: Inspect and test turn signal and hazard circuit flashers, switches, bulbs, sockets, connectors, terminals, relays, wires and LEDs; repair or replace as required.

Task 4: Inspect, test and adjust backup light and warning devices, circuit switches, bulbs, sockets, connectors, terminals and wires; repair or replace as required.

## Gauges and warning devices diagnosis and repair

Task 1: Diagnose the cause of intermittent, high, low or no gauge readings; determine needed repairs.

Task 2: Diagnose the cause of control area network (CAN) driven gauge malfunctions; determine needed repairs.

Task 3: Inspect, test and adjust gauge circuit sending units, sensors, gauges, connectors, terminals and wires; repair or replace as required.

Task 4: Inspect and test warning device (lights and audible) circuit sending units, sensors, bulbs, audible component, sockets, connectors, terminals, wires and printed circuits/control modules; repair or replace as required.

Task 5: Inspect and test electronic speedometer and odometer systems; replace as required; verify proper calibration for vehicle application.

## Related systems

Task 1: Inspect and test horns, horn circuit relays, switches, connectors, terminals and wires; repair or replace as required.

Task 2: Diagnose the cause of constant, intermittent or no wiper operation; diagnose the cause of wiper speed control and/or park problems.

Task 3: Inspect and test wiper motor, resistors, park switch, relays, switches, connectors, terminals and wires; repair or replace as required.

Task 4: Inspect and test windshield washer motor or pump/relay assembly, switches, connectors, terminals and wires; repair or replace as required.

Task 5: Inspect and test side view mirror motors, heater circuit grids, relays, switches, connectors, terminals and wires; repair or replace as required.

Task 6: Inspect and test HVAC electrical components, including A/C clutches, motors, resistors, relays, switches, controls, connectors, terminals, and wires; repair or replace as required.

Task 7: Inspect and test engine cooling fan electrical control components; replace as required.

Appendix C

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Sample Curriculum

Electrical/Electronics—Module A1

General Electrical Diagnosis 101

Goal:   Participants should understand the fundamentals of basic electrical theories and laws, basic schematic reading, and basic circuit protection.

Objectives:

Upon completion of this course participants should be able to:

- Read basic wiring schematics, identify symbols, and read single circuit electrical schematics

- Determine which schematic is related to a specific bus system

- Demonstrate knowledge of basic electrical theories and laws (including Ohm’s law)

- Inspect, test, and replace circuit breakers and fuses

- Diagnose the condition of fuses, circuit breakers, and switches

- Demonstrate ability to use digital multimeters and other test equipment

- Perform tests for voltage draws and drops using a digital multimeter

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 101 Schematics

- 101 Electrical Theory

- 101 Circuit Protection

- 102 Circuit Protection

- 103 Test Equipment

- 103 Test Equipment

Course Description:

Participants will receive classroom instruction where a qualified instructor will review the basics of electrical diagnosis including:  pertinent electrical theories and laws, schematic reading, safety issues, applicable tools/equipment, and circuit protection.  Participants should leave the course with a basic understanding of how electrical theories and laws and how that applies to general diagnosis.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  24 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer: Educational Data Systems, Inc

Subject Matter Experts: Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module A2

General Electrical Diagnosis 201

Goal:   Participants should understand more advanced electrical theory, know how to use testing equipment, demonstrate an advanced knowledge of schematics, and a basic knowledge of multiplexing, data communications, solenoids, and sensors, senders, and probes.

Objectives:

Upon completion of this course participants should be able to:

- Explain magnetic inductance, electrical theory of components, describe waveforms, and explain the operation of a suppression diode

- Draw and use wiring diagrams for troubleshooting and equipment assembly

- Use test equipment including: diagnostic software, ammeter, digital multimeters, computers, and lap scopes/oscilloscope/graphing multimeters

- Inspect, test, and replace resistors, diodes, and capacitors

- Replace solenoids and identify sensors, senders, and probes

- Diagnose circuit malfunctions and inspect wiring on engines

- Explain the function of the multiplex controller

- Read, interpret, and use ladder logic diagrams

- Troubleshoot multiplex controller and diagnose, inspect, and troubleshoot input and output circuits in multiplex systems

- Repair faults through reading ladder logic diagrams; check and replace multiplex control modules

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 101 Electrical Theory

- 201 Schematics

- 103 Test Equipment

- 201 Electronics & Data Communications

- 102 Switches, Relays & Solenoids; 201 Solenoids

- 201 Electrical Circuits; 201 Wiring, Connections & Connectors

- 201 Multiplexing

- 201 Multiplexing

- 202 & 203 Multiplexing

- 202 Multiplexing

Course Description:

Participants will receive classroom instruction where a qualified instructor will review more advanced aspects of electrical diagnosis including:  advanced electrical theories, testing equipment, advanced schematic reading, ladder logic diagrams, multiplexing systems, and circuits, solenoids, diodes, resistors, and capacitors.

Recommended Class Size:    4 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  16 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module B1

Battery 101

Goal:   Participants should be able to safely and properly handle, inspect, test, clean, and charge batteries.

Objectives:

Upon completion of this course participants should be able to:

- Safely handle batteries and acid

- Inspect and replace batteries

- Clean and check battery terminal connections

- Measure battery voltage/surface charge

- Check battery box construction for stress cracks/welds

- Measure battery acid specific gravity

- Perform battery load and capacity tests

- Charge batteries

- Jump start bus using jumper cables and auxiliary power supply

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 101 Batteries

- 101 Batteries

- 101 Batteries

- 101 Batteries

- 101 Batteries

- 102 Batteries

- 102 Batteries

- 102 Batteries

- 102 Batteries

Course Description:

Participants will receive classroom instruction where a qualified instructor will review bus battery basics including safety, testing, cleaning and inspection, replacement, charging, and jump starting a bus.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  34 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module C1

Starting Systems 101

Goal:   Participants should be able to identify starting system components, perform inspection, perform voltage drop tests, and replace starter circuit components.

Objectives:

Upon completion of this course participants should be able to:

- Inspect starters (motors and solenoids)

- Perform voltage drop tests on starter circuits using tester unit

- Replace starter circuit relays, solenoids, and switches

- Replace starter motor

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 101 & 103 Starting

- 101 Starting

- 101 Starting

- 101 Starting

Course Description:

Participants will receive classroom instruction where a qualified instructor will review the basics of starting system diagnosis and repair.  Emphasis will be placed on inspection, component identification and circuit component replacement.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  4 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

*Need vehicle to perform voltage drop

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module C2

Starting Systems 201

Goal:   Participants should be able to perform more advanced starting system diagnosis and repair including replacement of circuit wiring, distinguishing between mechanical and electrical causes, and complete starting system repair/replacement.

Objectives:

Upon completion of this course participants should be able to:

- Replace starter circuit wiring

- Distinguish between mechanical and electrical causes of starter problems

- Complete starting system inspection and diagnosis

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete.

- 201 Starting

- 201 Starting

- 201 Starting

Course Description:

Participants will receive classroom instruction where a qualified instructor will assist the participants in improving their skills with starting system diagnosis and repair.  Advanced repairs such as replacement of starter circuit wiring will be emphasized and participants will be taught to distinguish between mechanical and electrical causes of starter problems.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  4 to 8 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module D1

Charging Systems 101

Goal:   Participants should understand the basics of charging systems and all related components.

Objectives:

Upon completion of this course participants should be able to:

- Explain the purpose of charging systems and identify all related components

- Adjust or replace alternator components (belts, gears, fans, etc.)

- Remove and replace alternators

- Check and adjust voltage regulator set points

- Replace voltage regulators

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 201 Charging

- 101 Charging

- 101 Charging

- 102 Charging

- 103 Charging

Course Description:

Participants will receive classroom instruction where a qualified instructor will review the basics of charging systems including component identification.  Focus will be placed on adjusting, replacing, and removing alternators and checking, adjusting, and replacing voltage regulators.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  4 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module D2

Charging Systems 201

Goal:   Participants should be able to perform inspection, testing, diagnosis, and repair of charging systems.

Objectives:

Upon completion of this course participants should be able to:

- Diagnose the cause of various charging conditions and determine proper repair procedure

- Inspect overall charging system operation

- Inspect and repair connectors and wires in charging circuits

- Perform alternator output tests using tester unit

- Diagnosis charging systems using fault tree chart

- Perform alternator voltage drop tests using tester unit

- Inspect and troubleshoot power supply

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 201 Charging

- 201 Charging

- 201 Charging

- 201 Charging

- 201 Charging

- 201 Charging

- 201 Charging

Course Description:

Participants will receive classroom instruction where a qualified instructor will review on diagnosis, inspecting, testing, and troubleshooting charging systems.  Emphasis is placed on diagnosis and repair of the charging system as well as proper testing and inspection procedures.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  4 to 8 hours *(Troubleshooting portion with 3 students per Instructor)

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module E1

Lighting Systems 101

Goal:   Participants should be able to perform visual inspections of lighting systems components to determine necessary replacements.

Objectives:

Upon completion of this course participants should be able to:

- Read basic wiring diagrams

- Replace fuses

- Visually check wiring for interior and exterior lights

- Replace headlights, LED, florescent, and incandescent lights

- Replace interior or exterior lighting circuits

- Inspect electrical circuits

- Replace  ballast for interior lights

- Diagnose lighting systems and circuits

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 101 Schematics

- 101 Circuit Protection

- 101 Lighting

- 101 Lighting

- 102 Lighting

- 102 Lighting

- 103 Lighting

- 201 Lighting

Course Description:

Participants will receive classroom instruction where a qualified instructor will review reading wiring diagrams, performing visual inspections, and replacing various interior and exterior lights and lighting circuits.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  4 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module F/G 1

Wiring, Connections, and Connectors 101

Goal:   Participants should be able to demonstrate basic knowledge of wiring, connections, and connectors as it relates to gauges, warning devices, and related systems such as horns, wipers, and HVAC components.

Objectives:

Upon completion of this course participants should be able to:

- Read wiring diagrams related to specific components

- Replace connectors

- Assemble connectors

- Solder connections

- Splice connections

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 101 Schematics

- 102 Wiring, Connections & Connectors

- 102 Wiring, Connections & Connectors

- 104 Wiring, Connections & Connectors

Course Description:

Participants will receive classroom instruction where a qualified instructor will review the basics of wiring, connections, and connectors as it relates to gauges, warning devices, horns, wipers, and HVAC components.  Participants will receive instruction on related wiring schematics and how to solder and splice connections.

Recommended Class Size:    8 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  8 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module F/G 2

Gauges, Warning Devices, and Related Systems 201

Goal:   Participants should be able to use wiring diagrams to troubleshoot, diagnose, inspect, and repair/replace complex electrical circuits, use proper test equipment, and maintain various switches.  Participants should develop a better understanding of multiplex systems as well as data communication.

Objectives:

Upon completion of this course participants should be able to:

- Diagnose, inspect, and troubleshoot complex electrical circuit

- Replace/rewire electrical power circuits

- Remove and replace electrical connectors and terminals

- Repair interlock systems and replace current limiters/PTC

- Identify proper settings and calibrate address/dip switches

- Repair or replace magnetic, optical and/or half-effect proximity switches

- Use service tools and fault codes to diagnose communication link problems or repair systems

- Repair or create wiring harnesses

- Inspect, troubleshoot, and repair/replace CPU card, Gateway module, data communications network, data communications cable, and HCNC and DIO modules

- Use code scanner and adapter card to troubleshoot system

- Inspect, troubleshoot, and repair/replace doors, door wiring, and related components

- Demonstrate an increase understanding of multiplex systems including: PLC’s, logic diagrams, input and output cards, HCNC and DIO modules, multiplex software, I/O blocks, CPU cards, and multiplex controllers

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 201 Electrical Circuits

- 201 Electrical Circuits

- 201 Wiring, Connections & Connectors

- 202 Electrical Circuits

- 202 Electrical Circuits

- 202 Switches

- 202 Test Equipment

- 203 Wiring, Connections & Connectors

- 203 Electronics & Data Communication

- 203 Electronics & Data Communication

- 201 Doors

- 201 & 202 Multiplexing

Course Description:

Participants will receive classroom instruction where a qualified instructor will review how to use wiring diagrams to troubleshoot, diagnose, inspect, and repair/replace complex electrical circuits; proper use test equipment; and maintain various switches.  Emphasis is placed on developing a high understanding of data communications and multiplex systems.

Recommended Class Size:    4 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  8 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Electrical/Electronics—Module F/G 3

Gauges, Warning Devices, and Related Systems 301

Goal:   Participants should be able to maintain, diagnose, and repair fire suppression systems and apply advanced multiplexing knowledge to troubleshoot, inspect, and maintain multiplexing systems.

Objectives:

Upon completion of this course participants should be able to:

- Maintain fire suppression electrical systems (standards)

- Diagnose, repair, and replace VMU units

- Troubleshoot and maintain multiplexing systems

- Inspect/troubleshoot main bus controller and PMS modules

- Check/repair/replace control racks

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

- 301 Fire Suppression

- 301 Fire Suppression

- 301 Multiplexing

- 301 Multiplexing

- 301 Multiplexing

Course Description:

Participants will receive classroom instruction where a qualified instructor will review fire suppression system maintenance, diagnosis, and repair as well as multiplexing system troubleshooting, inspection, and maintenance.

Recommended Class Size:    4 to 1

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:  8 hours

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Student workbook, manuals, handouts, flow charts, power point, homework assignments

Instructor:

Course Developer:  Educational Data Systems, Inc.

Subject Matter Experts:  Contact APTA

Revision Dates:  8/31/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.