Training Syllabus to Instruct/Prepare for the ASE Transit Bus Transmission and Drive Train Test

Abstract: This Recommended Practice provides guidelines for establishing a standardized bus maintenance training related to the ASE certification program syllabus for transmission and drive trains used in transit buses and coaches.

Keywords: Automotive Service Excellence (ASE) H3, bus, certification, drive trains, training, transit, transmissions

Summary: This Recommended Practice allows users to instruct/prepare transit bus technicians and mechanics for the National Automotive Service Excellence (ASE) H3 Transit Bus Transmission and Drive train Test and to evaluate, develop or enhance current training programs for the diagnosis, repair and maintenance of transit bus transmission and drive train systems. Individual operating agencies should modify these guidelines to accommodate their specific equipment and mode of operation.

Scope and purpose: This Recommended Practice reflects the consensus of the APTA Bus Standards Program members in conjunction with transit labor organizations, including ATU and TWU, on the subject material, manuals and textbooks, test equipment, methods and procedures that have provided the best performance record based on the experiences of those present and participating in meetings of the program task forces and working groups. APTA recommends the use of this document by organizations that have a training department or conduct training for the maintenance of transit buses, organizations that contract with others for transit bus maintenance training, and organizations that influence how training for transit bus maintenance is conducted.

Contents

Participants

The American Public Transportation Association greatly appreciates the contributions of the Bus Transit Standards Transmissions and Drivetrains Working Group, which provided the primary effort in the drafting of this Recommended Practice.

At the time this standard was completed, the working group included the following members:

xx

1.	Learning environment	1

2.	Computer skills	1

3.	Course descriptions and objectives	1

4.	Exam requirements	1

5.	ASE test content summary	2

References	3

Definitions	3

Abbreviations and acronyms	3

Appendix A- Transit Bus Trans/Drive Train Learning

Co-Chair-Bob Hykaway ATU

Co-Chair-Dennis Cristofaro CTA      Edward Owens-TWU Local 234       Dan Engelkes-Rockford MT             Robert Romaine - TWU                   Hector Ramirez –TWU Local 100     John Burke - TWU Local 100           Jack Clark – TLC

John Schiavone - TLC                             Darryl Desjarlais-New Flyer Ind.      Mark Dalton-King Co Metro               David Gerber – ATU Local 85          Tony Pilewski – ATU Local 85         Donald Davis-Metro Minn-St Paul  John Webster – ATU Local 382       James Lindsey-  ATU Local 1277    Jeff Hunt – ATU Local 757               Joe Seitz-Maryland Transit

Ken Mall – EDSI

Brian Lester - EDSI

Objectives…………………………………………………………...…4

Appendix B- ASE Trans/Drive Train Task List………….………..11

Appendix C- Sample Curriculum………………….………………13

# Learning environment

For best application of this Recommended Practice, a combination of classroom lectures, mentoring, practical training and practice tests should be included in the training program.

# Computer skills

Basic computer skills are now a standard for transit bus technicians. Basic skills and knowledge in the operation of a computer in a Microsoft Windows environment is essential.

# Course descriptions and objectives

The Learning Objectives listed below have been developed through a Labor-Management Committee of Subject Matter Experts (SME).  The Learning Objective Levels represent 100 (Introductory), 200 (Intermediate), and 300 (Advanced).  Within each level the Learning Objectives are organized in the recommended order of instruction.  When a transit bus mechanic demonstrates proficiency in the learning objectives they should be capable of attaining ASE Transit Bus Technician Certification.

Modules A-1, A-2, and A-3:  Automatic Transmission 101, 201, 301

The objective of these courses is to familiarize the employee with the basics of automatic transmission components and operation, diagnosis, inspection, testing, and component repairs/replacements to restore the automatic transmission to OEM specifications.

Module B-1:  Drive Shaft and Universal Joints 101

The objective of this course is to familiarize the employee with the basics of drive shaft and universal joint components and operation, diagnosis, inspection, causes of failures, and needed repairs to restore all components to OEM specifications.

Module C-1:  Drive Axle 101

The objective of this course is familiarize the employee with the basics of drive axle diagnosis and repairs including inspections, cleaning, removing/replacing components, and restore the drive axle to OEM specifications.

# Exam requirements

The minimum acceptable grade to pass the course and all practical tests is 75 percent. Students must pass written tests with a minimum grade of 80 percent.

# ASE test content summary

| TABLE 1 Specifications for Transit Drive Train Test |  |  |
| --- | --- | --- |
| Content Area | Questions in Test | Percent of Test |
| A. Automatic Transmission Diagnosis and Repair | 28 | 70% |
| B. Drive Shaft and Universal Joint Diagnosis | 4 | 10% |
| C. Drive Axle Diagnosis and Repair | 8 | 20% |
| Total | 40 | 100% |

# References

National Institute for Automotive Service Excellence (ASE) website. http://www.ase.com/

# Abbreviations and acronyms

ABS	antilock brake system

APTA	American Public Transportation Association

ASE	Automotive Service Excellence

ATC	automatic traction control

ATU	Amalgamated Transit Union

DMM	digital multimeter

LED	light-emitting diode

OEM	original equipment manufacturer

PPE	personal protective equipment

RPM	revolutions per minute

TWU	Transit Workers Union

| Appendix A Transit Bus Transmission/Drive Train Learning Objectives |  |
| --- | --- |
| 101 Safety & Compliance |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper stall test safety procedures | A4 |
| Demonstrate proper disposal/recycling of fluids | A7, C3, C4 |
| Demonstrate proper disposal of filter | A7 |
| Explain importance of complying with manufacturer’s recommendations | ALL |
| 101 Theory & Use of Reference Materials |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of service manual | A3, A4, A9, A10, A13, A14, B1 |
| Describe importance of maintaining transmission fluid at correct level | A2 |
| Refer to manufacturer’s specs | A18, C3 |
| Check OEM manual for proper fluid type | A2 |
| 101 Procedures, Inspections and Testing |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of dipstick for checking fluid level (operating temp, level ground, gear selection) | A2 |
| Demonstrate proper use of shifter pad or other level indicators for checking fluid level | A2 |
| Visually check for transmission fluid leaks | A2 |
| Demonstrate proper use of drain pan | A7 |
| Check lines for rubbing or other damage | A10 |
| Visually inspect for leaks | C2 |
| Visually inspect bolt holes | C5 |
| Visually inspect sealant surfaces | C6 |
| 102 Safety & Compliance |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of safety equipment | A3 |
| Follow proper safety precautions | A5, A6 |
| Demonstrate proper use of personal protective equipment (PPE) | C2 |
| 102 Theory & Use of Reference Materials |  |
| Learning Objectives | ASE Task Reference |
| Check OEM manual for proper fluid, filter, and fluid capacity | A7 |
| Refer to manufacturer’s specifications | A2, A7, A16, A17, B3, C1, C2 |
| Demonstrate use of troubleshooting guide | A18 |
| Demonstrate ability to refer to manual for proper part number | C2 |
| 102 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of torque wrench and air pressure gauge | A7, A9, A10, A17, C4, C6, C7, C8, C9 |
| Demonstrate proper use of jacks and jack stands | A15 |
| Demonstrate proper use of handheld diagnostic reader to check level (LED) | A16 |
| Demonstrate proper use of seal installers and pullers | A18 |
| Demonstrate proper use of U-joint pullers and presses | B2 |
| 102 PROCEDURES, INSPECTIONS AND TESTING |  |
| Learning Objectives | ASE Task Reference |
| Check transmission fluid condition | A1 |
| Check shift cable operation and for proper lubrication | A6 |
| Apply proper torque for drain plug and filter housing bolts | A7 |
| Refill fluid to proper level | A7, A8 |
| Use of proper external seal and gasket hardware | A9 |
| Visually check for leaks | A10 |
| Drain and refill fluids | A16 |
| Visually inspect for fluids leaks, damaged holes and/or damaged threads | A17 |
| Check drive shaft for proper phase | B1 |
| Check for drive shaft damage | B1 |
| Check mounting flange/fasteners | B1 |
| Check U-joint wear | B1 |
| Check for proper lubrication | B1, B3 |
| Check for play | B3 |
| Check for rust | B3 |
| Check mounting fasteners/hardware for drive line | B3 |
| Check fluid level and condition in rear drive axle | C1 |
| Inspect and clean rear drive axle breather | C1, C2 |
| 103 SAFETY & COMPLIANCE |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper safety procedures | A1 |
| 103 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Refer to manufacturer’s specs for angle, torque, offset of transmission and flex plate | A15 |
| 103 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of mechanical tachometer | A4 |
| Demonstrate proper use of gasket sealant | A9 |
| Demonstrate proper use of pyrometer (temp gun) and analog thermometer | A12 |
| 103 PROCEDURES, INSPECTIONS AND TESTING |  |
| Learning Objectives | ASE Task Reference |
| Identify transmission fluid condition through sight and smell | A2 |
| Check cables, shift linkages | A6 |
| Visually inspect cooler, lines, breathers, filters and fittings | A10 |
| Inspect breather for clogs and contaminants | A11 |
| Visually check mounts and clamping hardware for damage | A15 |
| Check drive shaft alignment angle | B1 |
| Check output shaft | B1 |
| Check pinion shaft | B1 |
| Check slip joint and U-joint for proper lubrication and seals | B1 |
| 201 SAFETY & COMPLIANCE |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of personal protective equipment (PPE) | C4, C7, C9 |
| 201 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Explain causes and consequences of drive shaft vibrations | A1 |
| Evaluate oil analysis report | A2 |
| Follow manufacturer instructions for flushing machine | A10 |
| Refer to manufacturer’s specs for proper resistance, clearance and depth | A12 |
| Refer to manual for proper torque specs | C4 |
| 201 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of handheld diagnostic equipment | A1, A9, A13, A14 |
| Demonstrate use of diagnostic equipment for tachometer reading | A4 |
| Demonstrate proper use of pullers, extractors and installers | A10 |
| Demonstrate proper use of lifts, forklifts and cranes | A15, C4 |
| Demonstrate proper use of digital multimeter (DMM) | A9, A12, A13, A14, A18, A19, A20 |
| Demonstrate proper use of axle puller | C2, C6 |
| Demonstrate proper use of dial indicator | A17, B2, C6, C7, C8, C9 |
| Demonstrate proper use of wheel jack | C7, C8 |
| 201 PROCEDURES, INSPECTIONS & TESTING |  |
| Learning Objectives | ASE Task Reference |
| Perform stall test | A1 |
| Calibrate dipstick and demonstrate proper dipstick usage | A7 |
| Compare electronic (level sensor) reading to mechanical (dipstick) fluid level | A8 |
| Verify proper bolts being used and check for cracks or bends | A15 |
| Remove air lines | A15 |
| Demonstrate proper use of clamping hardware | A15 |
| Visually inspect flex plate | A15 |
| Check axle mounting hardware | C1 |
| Check for pinion play | C1 |
| Replace rear axle drive unit fluid and fill to proper level | C3 |
| Adjust speed sensor air gap | C7 |
| Visually inspect locking and nuts for damage | C7 |
| Check fluid level in differential | C7, C9 |
| Inspect wheel bearing end play, adjust to manufacturer’s specifications or replace as needed | C7, C9 |
| Learning Objectives | ASE Task Reference |
| Remove, replace and/or seal wear rings | C7, C9 |
| Visually inspect for abnormal wear and/or damage | C8 |
| Visually inspect locking and nuts for damage | C9 |
| Inspect planetary thrust washers and planetary gear for abnormal wear | C9 |
| 202 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Describe the uses of different seal types | A9 |
| Use manufacturer’s specifications to identify proper runout and engine-to-transmission mounting adapters | A16 |
| Refer to service manual for removal and replacement of axle and driveshaft procedures | C4 |
| 202 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of analog tachometer (RPMs) | A4 |
| Demonstrate proper use of depth gauge | A8, A9, A17 |
| Demonstrate proper use of flushing equipment | A10 |
| Demonstrate proper use of diagnostic equipment to check temperature circuit | A12 |
| Demonstrate ability to choose appropriate diagnostic tool | A18, A19, A20 |
| Demonstrate proper use of seal remover and installer | C7, C8, C9 |
| 202 PROCEDURES, INSPECTIONS AND TESTING |  |
| Learning Objectives | ASE Task Reference |
| Check related components for vibrations | A1 |
| Road test bus to determine shifting problems | A1 |
| Perform road test for lockup converter system test | A5 |
| Drain/flush torque converter and transmission cooler | A7, A10 |
| Demonstrate ability to use proper lines and fittings | A10 |
| Demonstrate ability to troubleshoot using known good parts when appropriate | A13 |
| Disconnect/Reconnect electrical cables/harnesses | A16 |
| Inspect seals and gaskets | A9 |
| Inspect wheel bearing end play and adjust to manufacturer's specifications | C1, C8 |
| Demonstrate ability to use proper sealing material | C4, C5, C6 |
| Inspect axle shafts for cracks | C5 |
| Inspect tone ring runout | C7 |
| Visually inspect tone ring for missing teeth and other damage | C7 |
| 203 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate ability to read and interpret schematics | A9, A13 |
| Demonstrate knowledge of drive line clearances | A13 |
| Demonstrate knowledge of vibrations in transmission and (differential) rear end | A13 |
| Demonstrate ability to use troubleshooting guides for drive train mounting problems | A14 |
| 203 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of pressure gauge set | A1, A3, A5 |
| Demonstrate proper use of laptop and diagnostic software package | A1, A4, A13, A14, A15, A19, A20 |
| Demonstrate proper use of diagnostic equipment for determining input speed of engine and turbine speed of transmission while in operation | A5 |
| Demonstrate proper use of temperature gauge or laptop to test temperature drop | A12 |
| Demonstrate proper use of pressure test equipment to check transmission cooler for leaks | A10 |
| 203 PROCEDURES, INSPECTIONS AND TESTING |  |
| Learning Objectives | ASE Task Reference |
| Inspect output speed sensor | A11 |
| Repair damaged holes and threads in housing | A16 |
| Describe the operation of multiplex interface | A20 |
| Demonstrate ability to troubleshoot multiplex interface | A20 |
| Check speed sensor for resistance and output | C7 |
| 301 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Describe the various means of data of communication between the drive train and other vehicle components | A20 |
| 301 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of angle gauge | B2 |
| Demonstrate proper use of dial caliper | A9 |
| Demonstrate proper use of offset gauge | B2 |
| Demonstrate proper use of spring tension gauge | A9 |
| 301 PROCEDURES, INSPECTIONS AND TESTING |  |
| Learning Objectives | ASE Task Reference |
| Check pinion shaft backlash | C1 |
| Evaluate transmission fluid analysis report | C3 |
| 302 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Explain the role of temperature and resistance | A14 |
| 302 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate ability to use laptop to check data communication link | A13 |
| Determine proper data interface cable to use for laptop | A13 |
| Determine proper translation module | A13 |
| Demonstrate proper use of a micrometer | C7, C9 |
| 303 THEORY & USE OF REFERENCE MATERIALS |  |
| Learning Objectives | ASE Task Reference |
| Describe the process of data networking between different systems (ABS, ATC, etc.) | A13 |
| Demonstrate ability to read hydraulic, pneumatic, electrical and mechanical schematics | A19 |
| 303 TOOL USAGE & DIAGNOSTICS |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate proper use of laptop for testing pressure | A3 |
| Demonstrate proper use of straight edge and feeler gauge | A17 |
| Demonstrate proper use of a breakout box to determine if problem software-related or hardware-related | A19 |
| 303 PROCEDURES, INSPECTIONS AND TESTING |  |
| Learning Objectives | ASE Task Reference |
| Demonstrate ability to reset fast adapt/adaptive shifting | A19 |
| Check communications cables | A20 |
| Demonstrate ability to interpret communications | A20 |

Appendix B:

ASE Transit Bus Drive Train Tasks

A. Automatic Transmission Diagnosis and Repair

Task 1:  Diagnose noise, vibration, and shifting problems; determine needed repairs.

Task 2: Check transmission fluid level; check dipstick calibration; diagnose fluid usage, leaks, and condition; determine needed repairs.

Task 3: Perform transmission pressure tests; determine needed repairs.

Task 4: Perform stall tests; determine needed repairs.

Task 5: Perform lock-up converter system tests; determine needed repairs.

Task 6: Diagnose mechanical control systems; determine needed repairs.

Task 7: Replace fluid and internal/external filter(s).

Task 8: Inspect, test, adjust, and/or replace retarder controls, valves, air lines, sensors, and components.

Task 9:  Inspect and replace external seals and gaskets.

Task 10:  Inspect, test, flush, transmission, transmission cooler and lines; inspect breathers, filters, and fittings; service as required.

Task 11: Inspect, test, reinstall or replace vehicle speed sensor.

Task 12:  Inspect and test transmission temperature circuit for accuracy; determine needed repairs.

Task 13:  Inspect, test, diagnose, adjust, repair, or replace electrical/electronic components including the transmission control module (TCM), electronic modulators, solenoids, sensors, relays, switches, lights, fuses/breakers, wiring and connectors.

Task 14:  Inspect, replace, and align drive train mounts.

Task 15:  Remove and replace transmission; inspect flex plate.

Task 16:  Inspect engine block, flywheel housing, and transmission mating surfaces; check runout; check engine-to-transmission mounting adapters; determine needed repairs.

Task 17:  Inspect, test, repair, or replace electronic shift selectors, switches, displays and indictors, and wiring harnesses.

Task 18: Diagnose automatic transmission and retarder problems using appropriate diagnostic tools and software, procedures, and service information/flow charts; check and record diagnostic trouble codes; clear codes; interpret digital multimeter (DMM) readings;

Task 19:  Diagnose automatic transmission problems caused by data link/bus interfaces with

the transmission control module (TCM); identify electrical interference problems; determine needed repairs.

Task 20:  Inspect, adjust, service, repair, or replace power take-off assemblies and controls.

B. Drive Shaft and Universal Joint Diagnosis and Repair

Task 1: Diagnose drive shaft and universal joint noise and vibration problems; determine cause of failure and needed repairs.

Task 2: Inspect, service, or replace driveshaft, slip joints, yokes, drive flanges, universal joints, and vibration dampers; phase drive shaft yokes.

Task 3: Measure drive line angles; determine needed adjustments.

C. Drive Axle Diagnosis and Repair

Task 1: Diagnose rear axle drive unit noise, vibration, and overheating problems; determine needed repairs.

Task 2: Check and repair fluid leaks; inspect and replace rear axle drive unit cover plates, gaskets, breathers, magnetic plugs, and pinion seals.

Task 3: Check rear axle drive unit fluid level and condition; determine needed service; add proper type of lubricant.

Task 4: Remove and replace differential carrier assembly, check ring and pinion backlash, inspect rear axle housing mating surfaces; determine needed repairs.

Task 5: Remove, inspect, and replace axle shafts.

Task 6: Remove and replace rear wheel hub assembly; inspect ABS tone/exciter ring and wheel speed sensor; determine needed repairs.

Task 7: Diagnose wheel bearing noises and damage; determine needed repairs.

Task 8: Clean, inspect, lubricate, and replace wheel bearing cones and races (cups); clean and inspect locking plates and nuts; replace seals, wear rings, and axle flange gasket; adjust rear wheel bearings.

Task 9: Inspect, adjust, repair, or replace planetary axle assemblies including case, idler pinion, pins, gears, thrust washers, shims, seals, cover, and springs.

Appendix C:

Sample Curriculum

Transmission/Drive Train-Module A1

Automatic Transmission 101

Goal:   Participants should understand the basics of automatic transmission components and operation.

Objectives:

Upon completion of this course participants should be able to:

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

Course Description:

Participants will receive classroom instruction where a qualified instructor will go over the basics of Automatic Transmission components and operation including: pertinent theory, component identification, safety issues, applicable tools/equipment, and basic testing.  Participants should leave the course with a basic understanding of how Automatic Transmissions operate and relevant components to operation.

Recommended Class Size:    8-12

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Instructor:

Course Developer:  EDSI

Subject Matter Experts:  Contact APTA

Revision Dates:  9/10/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Transmission/Drive Train—Module A2

Automatic Transmission 201

Goal:   Participants should understand basic troubleshooting techniques for a transit bus automatic transmission through an enhanced understanding of theory and proper operation.

Objectives:

Upon completion of this course participants should be able to:

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

Course Description:  Participants will receive instruction from a qualified instructor regarding basic troubleshooting and preventive maintenance procedures for an automatic transmission including the identification of defective/worn components and probably causes of failure.  Emphasis will be placed on inspection and testing procedures.

Recommended Class Size:

Pre-requisites: (previous module and/or demonstrated experience)

Participants should have taken Automatic Transmission 101 or have equivalent experience.

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:

Target Audience:  Anyone successfully completing Automatic Transmission 101 or demonstrating equivalent knowledge/experience.

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Instructor:

Course Developer:  EDSI

Subject Matter Experts:  Contact APTA

Revision Dates:  9/10/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Transmission/Drive Train—Module A3

Automatic Transmission 301

Goal:   Participants should be able to perform all necessary actions required to restore the automatic transmission to OEM specifications.

Objectives:

Upon completion of this course participants should be able to:

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

Course Description:  Participants will be led by a qualified instructor through the process of restoring the automatic transmission to OEM specifications.  This includes instructor demonstrations and participant practice.  This portion is highly interactive and should employ hands-on learning activities.

Recommended Class Size:

Pre-requisites: (previous module and/or demonstrated experience)

Participants should have taken Automatic Transmission 201 or have equivalent experience.

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:

Target Audience:  Anyone successfully completing Automatic Transmission 201 or demonstrating equivalent knowledge/experience.

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Instructor:

Course Developer:  EDSI

Subject Matter Experts:  Contact APTA

Revision Dates:  9/10/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Transmission/Drive Train-Module B1

Drive Shaft and Universal Joint 101

Goal:   Participants should understand the basics of drive shaft and universal joint components and operation and how to troubleshoot and repair them to OEM specifications.

Objectives:

Upon completion of this course participants should be able to:

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

Course Description:

Participants will receive classroom instruction where a qualified instructor will go over the basics of drive shaft and universal join components and operation including: diagnosis, inspection, causes of failure, and repairs.

Recommended Class Size:    8-12

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Instructor:

Course Developer:  EDSI

Subject Matter Experts:  Contact APTA

Revision Dates:  9/10/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.

Transmission/Drive Train-Module C1

Drive Axle 101

Goal:   Participants should understand the basics of drive axle components, operation, diagnosis, and repairs to restore the drive axle to OEM specifications.

Objectives:

Upon completion of this course participants should be able to:

Job tasks learning objectives/OJT checklist:   Will be tied back to Transit Training Guidelines.  EDSI to complete

Course Description:

Participants will receive classroom instruction where a qualified instructor will go over the basics of drive axle components and operation including: diagnosis, inspection, cleaning, and repairs to fully restore the drive axle to OEM specifications.

Recommended Class Size:    8-12

Pre-requisites: (previous module and/or demonstrated experience)

Delivery Method (e.g. Lecture, Hands on, On-line, Lab):  Hands-on and Classroom

Course Duration:

Target Audience:  All new and existing mechanics

Classroom Equipment and Supplies:

Notepads, pens/pencils, flip chart or white board (and markers), chart markers, classroom, laptop, projector, highlighters, note cards, and name cards

Course Materials, Training Aids, and References:

Instructor:

Course Developer:  EDSI

Subject Matter Experts:  Contact APTA

Revision Dates:  9/10/2010

Follow Up:  Most recent revision should be sent to committee for feedback

Instructor and Course Evaluation:  Local course evaluation sheets should be used if present.